PRODUCT CARD SAO
Cat. no. 607

Swing dimensions:
Length: 2.6 m
Width: 2.6 m
Height: 1.6 m
Minimum space: 5.2 m diameter circle
Age group: 0 to 14 years
Free fall height: 1.0 m

Guidelines for materials and technology of construction of the device.

Main components made of 88.9 mm diameter tubes, handles made of 42.4 mm diameter tubes. All structural elements made of acid-resistant steel in grade 0H18N9. Foundations are made as concrete footings placed at a depth of 0.6 m. Seat top made of EPDM.  Rubber membrane made of 10mm thick reinforced rubber. The connection of the ropes to the steel structure and the rubber membrane is provided by polyamide thimbles. Ropes of 16mm diameter, polypropylene, twisted, reinforced with galvanised steel strings. The fittings are made of polyamide or aluminium. Fasteners, i.e. bolts, screws made of stainless or acid-resistant steel.

The fall area of the equipment should be made on a surface in accordance with PN EN 1176- 1:2017. The possibility of access by heavy construction equipment is required for the installation.

For the safety of children and the quality of the equipment, it is required that the equipment is certified in accordance with standards PN EN 1176-1:2017, PN EN 1176-3:2017, PN EN 1176-11:2014-11 issued by a system accredited by the National Accreditation Centre or the national accreditation body of the other Member States in accordance with Regulation (EC) No 765/2008 of the European Parliament and of the Council of the European Union."